At anchor in the Pacific Northwest.

A lazy broad reach, southbound across the Gulf.

Argent is a tough blue water cruising boat, built to endure the rough stuff with a

solid fiberglas hull and beefy rigging. Her mass and ideal ballast to displacement

ratio gives an easy motion in lumpy, confused water. Modern underbody with

cutaway forefoot and famous “Brewer bite” aft reduces wetted surface,  providing

a slippery, more easily driven hull as compared to full-keel vessels of similar type,

while still allowing her to track like a freight train on any point.

No boat in her category will sail better than a proper Jason 35. This design has

developed something of a cult following, deservedly so. If you are not yet familiar

with it, a Google search will get you to lots of information about these boats, and

check out also Ferenc Mate’s book: Best Boats to Build or Buy.

Again, the hull is solid fiberglass. There is no core of any type, and the heavy-

duty layup is entirely of woven rovings. There are no chopped strand mat layers.

Following Miller Marine’s production run, the molds ended up in Ontario where

some boats were produced. I do not know what the construction specifics of

those later boats might be, just be aware that not every Jason 35 was by Miller.

Two bladed prop aligns with the keel when sailing, effectively disappearing to the

water flow and significantly reducing resistance vis-à-vis a three bladed propeller.

I’m always surprised at my boat’s ability to ghost along in a light air.

Some of Argent’s gear:

Triple reef mainsail, 90% yankee, 120% genoa, two sails for inner forestay (one

with full length hoist is good to 35 knots, shorter one serves as storm jib), storm

trysail with dedicated mast track, genniker with pole and snuffing sock, riding

sail (rigged to hoist with topping lift). Lewmar winches, dual-speed primaries.

Repowered with new Volvo MD2030C engine, new prop, new propeller shaft,

new shaft bearing and new PSS shaft seal.

37 pound Max bower anchor (this bulldog has earned my trust) with 300 feet

5/16” hi-test chain, Simpson-Lawrence double action lever (manual) windlass,

35 pound CQR anchor for spare or doubling (hasn’t been in the water yet), 18 lb

Danforth lunch-hook aft (nylon rode feeds from spool in lazarette), removable

shore line spool rigs at stern pulpit.

Wheel steering (Edson pedestal and quadrant), with emergency tiller stowed in

lazarette. Ritchie Globemaster SP-5 compass mounted on the pedestal. Autohelm

ST4000 (wheel mounted) autopilot. Signet wind, depth, speed and log instruments.

Furuno 1620 radar, display pivots for helmsman’s use under way. Icom IC-M58

VHF radio. Sitex EZ-7 loran unit. Seriously! Might as well leave it hooked up as

long as they keep broadcasting signals.

Nine bronze Perko opening portlights.

Dickinson Newport diesel cabin heater, properly vented, with six gallon fuel tank.

Adler Barbour “Coldmachine” refrigeration. Gimballed Kenyon propane galley stove

with two burners and oven. Double stainless steel sinks.

Statpower “Truecharge 20+” multi-stage battery charger. Prowatt 400 modified

sine-wave inverter. Exeltech XP250 true sine-wave inverter – needed for powering

variable speed tools, or for “buzzlessly” operating an AC stereo system.

The view from the top, courtesy of my nifty hand-crafted mast track ladder. Don’t

try this at home, kids – it’s essentially an unprotected free-climb all the way up.

But it does allow me to take care of those little chores aloft without needing to

find an assistant.

Argent’s rig is simple and sturdy. Keel-stepped mast, single spreaders with dual

lower shrouds, all stays and shrouds (except the inner forestay) attach to the

hull with external chainplates. Inner forestay attaches at the deck with a quick-

release hook so it can be stowed out of the way if you want to, e.g., short tack to

weather rigged as a sloop. Note that the mast is stepped in accordance with the

revised plan, i.e. 18” forward of its position as originally specified. You’ll be glad

of this. Early Jasons rigged in the old configuration proved to have a rather severe

weather helm.

Out with the old . . .                                     . . . in with the new.

The old, slow-cranking, raw-water cooled, two cylinder Volvo and its astoundingly

massive flywheel seemed like a sort of relic from a forgotten age. I came to call it

Mr. Smelly. I rigged a bypass valve at the intake so I could, whenever possible,

flush it full of fresh water before shutting down. Still, the thing was gradually rotting

away from the inside out, in the manner of all raw-water cooled engines that stay

in salt water. The new three cylinder Volvo MD2030C works at around 3000 rpm,

rather than 1950, and so runs much more smoothly. Best of all, there’s fresh water

coolant circulating in a closed system. Even so, I stayed in the habit of running fresh

water through when shutting down, so salt water wouldn’t be sitting for long periods

in the heat exchanger and water lift tank. The new engine’s ticker is currently at

1220 hours.

Teak and Honduran mahogany in the interior. Vee-berth forward, head to port,

closet and drawers starboard, opening portlights all around.

View aft. Nav station is to starboard, galley opposite.

The galley. Microwave oven is for use with shore power only. Outboard sink has

two spigots and foot pumps, one drawing tank water through a filter and the

other fitted with a valve to select raw tank water or seawater.

Diesel heater puts out more BTUs than you’ll need, even at lowest flame.

Why I chose the Jason 35.

Jasons are rare birds, not often seen on the market. I’ve always felt really lucky

that one happened to come available during the time that I was searching for my

ideal sailboat. Some of its features to be thankful for:

Modern underbody.

Lots of salty looking boats in the offshore cruising category are based on designs

from the nineteenth century, complete with the usual full keel. The fellow who

originally commissioned the Jason design wanted a full keel, but as Ted Brewer

put it: “I pared away everything but the illusion.” This boat essentially has a long

fin and skeg arrangement, which means considerably less friction, so less power

is needed to move it, and the boat is also handier in close-quarters maneuvering,

like when you’re getting in and out of your slip. But still, slings are not required

for taking this boat out of water – drydocks or marine railways will do fine, and

you can sit it on a tide grid if you need to.

Encapsulated ballast.

The keel is an integral part of the fiberglass hull structure, and all the lead is

contained within it. Many boats have their ballast cast into shape as the actual keel,

which is then bolted to the bottom of the fiberglass hull. With that you get to worry

about corrosion eventually weakening the bolts to the point where the ballast keel

might part company with the boat, no doubt at the most inopportune time.

Skeg-hung rudder.

Supported at both top and bottom, it is able to withstand MUCH greater forces

than the spade rudders which are occasionally seen on boats of this type. And I

like the way the Jason’s rudder has its greatest area down low where it matters

most, too.

Prop in aperture.

You’ll really appreciate this feature when the man on watch’s attention wanders

and a large piece of drift rolls under your boat. Sail-drive units, and props on

straight shafts that emerge from the hull in the space between a fin keel and

rudder, are quite vulnerable in this situation. And remember, spade rudders are

equally in danger here.

Quality hull construction.

The Jasons hulls were hand laid using woven fabrics only. Why is this important?

There are lots of ways of combining glass fibers with plastic resin to make a boat.

The choice of what materials and methods to use is up to the manufacturer, and

is usually dictated primarily by the intended price point and desired profit margin.

Two types of glass fabric are typically used to build laminate thickness, the more

expensive having long strands woven into various weights of cloth, and a much

cheaper type – chopped strand mat – made up of short strands lying flat in random

directions, held together for ease of handling by a bonding agent which dissolves

when soaked with resin. Fabrics woven of long, directional fibers are much stronger,

and produce laminates with higher glass-to-resin ratios as well – it’s the glass which

provides the strength, the resin simply bonds it all together. Woven glass results in

MUCH tougher – although more expensive – boats. Chopped strand mat layups will

fail under far less stress, even when thicker and heavier than those of woven fabric.

The different fabrics can be used in combination. Layers of woven might alternate

with chopped strand, or there might be nothing but chopped strand finished off

with a layer of woven, which will of course be the visible one. Also, some type of

core layer might be sandwiched inside the glass in some areas, in order to make a

sufficiently rigid hull while using less of the heavier and more costly glass fabrics.

But further reducing its ability to withstand stress, of course, and introducing the

risk of eventual delamination into the whole bargain as well.

Whichever fabrics are used, the man-hours needed to properly wet out layer after

layer will be a huge part of the cost (good reason to “offshore” the manufacture of

fiberglass boats), and so the cheapest method of all was developed, chopper-gun

construction. Here, short fibers are combined with resin right at the nozzle of what

is essentially a big spray gun while the laminator stands back and aims the resulting

slurry into the mold. Cuts labor costs to a minimum, but the result will be a very

resin-rich layup, and not even as strong as one properly hand-laid of chopped strand

mat. Again, a chopper-gun layup could be combined with fabric, most likely a single

woven layer visible on top.

The point of all this is, you might drill a hole in the bottom of your boat to install a

through-hull fitting and discover the laminate is an inch and a half thick there, and

then think your boat is ever-so-tough, but this may not be the case. (In fact, if it were

that thick, I’d say the odds are very good there was a chopper-gun involved.) Its hull

may be considerably weaker than one less than half as thick.

Find a copy of Lin & Larry Pardey’s book The Capable Cruiser, and study their report

on the 1982 Cabo San Lucas disaster. Quite a revealing story is told by the varying

degrees of damage sustained by the many yachts that were blown onto the beach

during that storm. There are photographs nicely contrasting the differences between

quality construction and cost-cutting cheapos. Some hulls lay on their bilges high

and dry but still repairable, while others were completely and utterly broken apart,

their insufficiently wet out layers of chopped strand glass revealed for all to see.

Beefy rigging.

Argent’s keel-stepped mast has a somewhat larger section than originally specified

for the Jason 35s, and the standing rigging is one step larger. I find this reassuring.

The chainplates, bolted to the exterior of the hull, cannot cause the leaky rotting kind

of problems associated with the type that pass through the deck and fasten to interior

bulkheads.

Cutter rig.

There are a number of advantages to the cutter rig, especially for offshore boats.

When close reaching and beating the staysail – particularly rigged like Argent’s

with boom and traveler – can be set to form an ideal slot, sending a good rush of

air across the mainsail’s luff, increasing its drive. Two jibs in the fore-triangle give

more options for shortening sail, to the point that you may feel no need to put up

with the disadvantages (probable equipment failure, reduced sail efficiency) of a

roller-furling jib. In heavier winds, flying just the staysail and reefed main keeps

the boat well balanced, unlike a shortened-down sloop whose reefed main and

small heavy weather jib out at the bow combine to pull the center of effort forward,

creating a troublesome lee helm.

No cosmetic teak decking.

Weathered teak has a nice traditional look, but when slats of thin wood are screwed

into a balsa or plywood cored deck you get hundreds of possible entry points for water

to find its way into the core and begin the process of rotting.

Not made in Taiwan.

The yacht brokers started calling those oriental imports “Taiwan teak palaces”, and

the appeal of all that wood trim up in the cockpit and in the fine joinery of the below

deck accommodation cannot be denied. Great marketing tool – makes it so much

easier for a prospective captain to sell the Mrs. on the whole idea of getting a sailboat.

I remember, when I was in quest of my dream boat, viewing a particular one of

these, a cutter of “traditional” styling. It had all the cute design cues that give those

“cartoon” boats that came out of Taiwan in the 1980s the salty, shippy look of vessels

that plied the oceans in previous centuries. And a stunning interior for sure, but very

little effort was required to get a peek at the less accessible areas and see, for example,

a few instances of fiberglass bulkhead tabbing that had fallen away when still wet and

then cured that way, hanging uselessly in space, to be shipped off just so to waiting

customers in America. It looked to me like solid evidence of questionable production

values, generally shoddy workmanship and nobody on the crew at all hired to do quality

control inspections. I could scarcely believe my eyes. Did they think no one would ever

look beyond the lovely cabinetry? That no one might poke their head all the way into

the forepeak and deep down into the lazarette? Seeing such things should make you

wonder about the quality of the hull lamination, too – was this manufacturer maybe

skimping on resin, was his hurried crew perhaps not completely wetting out all the

layers of chopped mat?

Scenes from a casual cruising life.

Wish you were here . . .

Where I always drop the hook when at Port Ludlow. I dislike anchoring in more

than 30 feet of water. Too much chain to haul up otherwise.

Reaching across Haro Strait for Speiden Channel.

Preparing breakfast after a blustery night at Silva Bay, I watched a group of three

trawler yachts heading out to “test the water”, whether intending for Vancouver

or northbound I don’t know. Half an hour later, as I was preparing to bring up the

hook, these boats returned to the protection of the anchorage, the old ladies out

on the bows calling to each other: “Four hours of that? NO THANKS!”

So this demonstrates another advantage of cruising in a sailboat. Look at this

picture from an hour or two later, it doesn’t seem all that rough does it? Heavy

texture but hardly a whitecap in sight. We’re loping along on a broad reach north

for Malaspina Strait now, a nice press of wind in the sails preventing the incessant

rolling a motor yacht would experience in these leftover seas.

At the only reasonable anchorage in Hotham Sound, another sailboat already

filled the one really good spot. So, here we are in about 70 feet and from the

sounds of things down there the anchor’s in a field of boulders. Really nice place

though, you should see the waterfall.

You have to look closely to pick out Argent, all by herself in this lovely Desolation

Sound anchorage. The photo’s here just to prove that it’s possible, with a little

effort, to find solitude even in the height of the season at the most popular of

yachting destinations. Actually, I think it was more a matter of luck.

It’s totally a different story once you get north of Port Hardy though – sometimes

you even start hoping for a little company. I’d wanted to bring my sailboat into

this Nigei Island anchorage ever since first having visited the place several years

earlier, during an epic sea kayak expedition.

A low tide view of the double-dogleg entrance to the bomb proof anchorage inside

Murray Labyrinth. You’ll probably want to creep in here at low water, very slowly,

with crew standing watch at the bow.

Just a couple miles before reaching the Labyrinth, I noted several yachts squeezed

together in the Skull Cove anchorages. I figured I’d find a similar situation at my

destination – the route in is well described in at least one guidebook, after all. But

there was not a soul around, and nobody came in during the few days I stayed, no

doubt all due to the perceived danger of the entrance. That’s fine with me.

One guidebook author has dubbed this place Five Window Cove. Nice anchorage.

Long but easy entrance, completely enclosed, and you can put it in shallow water.

One of the realities of cruising the Rain Coast, as viewed from the protection of

the companionway dodger. When this regime settles in, you soon learn to make

full use of every “two-hour window” (as the locals call them). We’re anchored

deep inside a little inlet that winds into an island group from the open Pacific. This

waterway and its adjuncts are tagged with no names on the charts, and there were

to my knowledge no yachting guidebooks describing the place. I therefore claimed

the prerogative and bestowed “Hermit Cove” upon this part of it, not in reference to

the total solitude, rather for a particular variety of thrush whose forest-filtered calls

echoed through the misty air there. In like manner, the basin behind that little island

across the way has been named Loon Lagoon.

Hermit Cove turned out to be an ideal place to wait out the gales – perfect depth for

my tastes, good bottom and protected all around. Here’s a deal: buy my boat and I’ll

tell you how to sneak inside the inlet, and show you the best route to avoid the charted

rock that sits in the middle of the entrance to this cove. And then you can name it

whatever you like.